Potential Resource Issue:

Water quality compliance.

Project Nexus:

Project structures, operations, maintenance activities, and Project betterments could affect water quality.

Potential License Condition(s):

- Instream flow releases

- Best management practices (BMPs)

- Spill Prevention Control Countermeasure (SPCC) Plan

Study Objective(s):

Characterize physical, chemical, and bacterial water quality conditions in the bypass reaches and the peaking reach, comparison reaches, and Project reservoirs and diversion pools and compare to the Central Valley Regional Water Quality Control Board (CVRWQCB 1998) Basin Plan objectives and water quality standards.

Extent of Study Area:

The study area will include bypass and comparison reaches, the peaking reach, Project reservoirs, and diversion pools (Tables AQ 11-1 and AQ 11-2, Maps AQ 11-1 and AQ 11-2).

Study Approach:

Water Quality Sampling Field Program

The following describes the water quality sampling field program which includes the collection of (1) in-situ water quality measurements; (2) general water quality samples; (3) fecal coliform samples; (4) fish tissue samples; and (5) contingency water quality and/or macroinvertebrate samples.

In-situ Field Measurements

- Collect in-situ water quality measurements at sampling locations listed in Table AQ 11-1 (i.e., bypass reaches, the peaking reach, Project reservoirs, and diversion pools) using a YSI meter.  Samples will be collected once during the spring runoff (April or May, access permitting), and once during the summer low flow or base flow period (August or September).  Pre- and post-sampling calibration of in-situ instrumentation will be conducted following the manufacturer’s instructions.

- Monitor in-situ water quality parameters at bypass reaches and the peaking reach, including dissolved oxygen (DO), pH, specific conductance, and water temperature.  In-situ parameters will be measured in a representative location of the stream.

- Monitor in-situ water quality parameters at all Project reservoirs and diversion pools including DO, pH, specific conductance, water temperature, and Secchi depth.  Reservoir profile analysis will be conducted at sampling locations in French Meadows Reservoir, Hell Hole Reservoir, and Ralston Afterbay.  Collect reservoir profile in-situ measurements of water temperature and DO at 1-meter (m) depth intervals to determine if thermal stratification is present.  If a thermocline is present, the water quality parameters will be measured below the thermocline at 2-m intervals to the bottom of the reservoir.

- Document if the results of the in-situ monitoring meet the water quality objectives identified in the Basin Plan (CVRWQCB 1998).

General Water Quality Sampling

- Collect general water quality samples at sampling locations listed in Table AQ 11-1 and depicted on Map AQ 11-1.  Samples will be collected once during the spring runoff and once during the summer low flow or base flow period.

- Collect water quality samples in bypass reaches and the peaking reach in a representative portion of the stream channel, using methods consistent with the Environmental Protection Agency (EPA) 1669 sampling protocol Sampling Ambient Water for Trace Metals at EPA Water Quality Criteria.  Water quality samples collected from streams will be analyzed for the parameters listed in Table AQ 11-3, which include general parameters, total Mercury, a suite of dissolved metals, and total and fecal coliform.

- Collect water quality samples from Project reservoirs and diversion pools at the surface and immediately below the thermocline, where present.  If the Project reservoir or diversion pool is not thermally stratified, then water quality samples will be collected at mid-depth of the reservoir.  Water quality samples for laboratory analysis will be collected using a Teflon® Kemmerer style sampler to ensure integrity of the sample collected from depth.  Water quality samples collected from reservoirs will be analyzed for the parameters listed in Table AQ 11-3, which include general parameters, hydrocarbons, total Mercury, a suite of dissolved metals, and total and fecal coliform.  Laboratory analysis for hydrocarbons will be conducted only on water quality samples collected from French Meadows and Hell Hole reservoirs, where motorized boating typically occurs.

- Water quality samples will be decanted into laboratory-supplied sample containers.  The sample containers will be labeled with the date and time that the sample is collected, sampling site or identification label.  The sample container will be preserved (as appropriate), stored and delivered to a State-certified water quality laboratory for analyses in accordance with maximum holding periods.  A chain-of-custody record will be maintained with the samples at all times.  The sampling site location will be recorded using a Global Positioning System (GPS) unit and the coordinates will be recorded in a field log book.

Coliform Sampling

- Conduct total and fecal coliform sampling to determine if study waters meet Basin Plan (CVRWQCB 1998) objectives for contact recreational activities.  Samples will be collected at near-shore locations adjacent to recreation facilities at Project reservoirs and along bypass reaches where substantial contact recreation (swimming, fishing, rafting, etc.) occurs.  Fecal coliform sampling locations are listed in Table AQ 11-2 and depicted on Map AQ 11-2.  In accordance with the fecal coliform sampling protocols, samples will be collected no less than five times within a thirty-day period between July 4 and Labor Day and will include one holiday weekend.  If early summer weather indicates that the July 4 weekend will be a large recreational weekend, sampling will include the July 4 weekend in preference to Labor Day.

Laboratory Analysis and Reporting

- Provide water quality samples collected during the field program to a State-certified laboratory approved by the State Water Resources Control Board (SWRCB) for chemical analysis.  The laboratory will report each chemical parameter analyzed with the laboratory method detection limit, reporting limit, and practical quantification limit.  The laboratory will attempt to attain reporting detection limits that are at or below the applicable regulatory criteria.  The parameters to be analyzed by the analytical laboratory are provided in Table AQ 11-3.

Fish Tissue Analysis

- Conduct a screening level study of methyl mercury concentrations in sport fish muscle tissue in consultation with the Aquatic Technical Working Group (TWG).

- Collect a total of 10 native (nonhatchery) sport fish of edible size, if present, from each of the following locations: French Meadows Reservoir, Hell Hole Reservoir, Middle Fork Interbay, Ralston Afterbay, and one location in the Middle Fork American River below Ralston Afterbay.

  - Submit fish to an analytical laboratory for individual fish muscle tissue analysis (fillets).  Field procedures will be consistent with those outlined in California Environmental Protection Agency (Cal EPA) (2005) and those used at the Department of Fish and Game Marine Pollution Studies Laboratory at Moss Landing (Method # MPSL-102a).

  - Larger fish and species with greater potential for bioaccumulation will be targeted.  Five fish each of two different species will be collected from each location based on the following priority ranking.  The two species present with the highest priority ranking will be collected (1 = highest priority):

    - 1) bass

    - 2) pikeminnow

    - 3) lake trout

    - 4) brown trout

    - 5) rainbow trout

  - Handle fish per Method # MPSL 102a.  Use clean nylon nets and polyethylene-gloves.  Record the species, fork length, and weight of each fish.  Place fish in Teflon sheets and into zipper-closure bags and place immediately on ice for delivery to the analytical laboratory.

- The analytical laboratory will analyze muscle tissue from each fish for concentrations of methyl mercury in accordance with the General Protocol for Sport Fish Sampling and Analysis developed by the Cal EPA (2005) and with methods comparable to those used at the Department of Fish and Game Marine Pollution Studies Laboratory at Moss Landing.

Contingency Sampling

Water Quality

- Consult with the Aquatic TWG if results from the general water quality sampling program indicate a potentially adverse water quality condition.  If appropriate, develop an additional sampling program to further investigate the cause or source of the potentially adverse water quality condition.

Fish Tissue Analysis

- If methyl mercury in fish tissue exceeds the OEHHA guidelines of 0.08 ppm (Cal EPA 2005; Klasing and Brodberg 2006) during the initial sampling, the Aquatic TWG will be consulted concerning the need for additional sampling.  If additional sampling is deemed appropriate, a sampling protocol will be developed.

- Fish tissue sampling may be expanded in the event water quality sampling identifies locations that are of potential concern due to the presence of levels of total Mercury that exceed the EPA Freshwater Aquatic Life criteria as described in 40 CFR §131.  If this occurs, the Aquatic TWG will be consulted concerning the need for fish tissue sampling.  If fish tissue sampling is deemed appropriate, a sampling protocol will be developed.

Benthic Macroinvertebrate Sampling

- Implement California Stream Bioassessment Procedures to quantify benthic macroinvertebrate (BMI) species compositions, abundance, and distribution at locations where potentially adverse water quality issues may exist or are identified by the water quality sampling program (see AQ 3 – Macroinvertebrate Technical Study Plan (TSP)).  Develop sampling to address potential water quality issues in consultation with the Aquatic TWG.

Schedule:

| Date | Activity |
| --- | --- |
| May 2007 | Collect water quality samples |
| August and September 2007 | Collect coliform samples |
| September 2007 | Collect water quality samples |
| October 2007 through January 2008 | Complete data analysis and prepare draft report |
| February 2008 | Submit draft report to the Aquatic TWG |
| February and March 2008 | Aquatic TWG 60 day review and comment period |
| April through June 2008 | Resolve comments and prepare final report |
| July 2009 | Submit final report to the Aquatic TWG and Plenary |
| September 2008 through June 2009 | Conduct contingency water quality related studies, data analysis and report production |

References:

- California Environmental Protection Agency (Cal EPA).  2005.  General Protocol for Sport Fish Sampling and Analysis.  Pesticide and Environmental Toxicology Branch, Office of Environmental Health Hazard Assessment, California Environmental Protection Agency.  December 2005.

- Klasing, S. and R. Brodberg.  2006.  Draft Development of Guidance Tissue Levels and Screening Values for Common Contaminants in California Sport Fish: Chloradane, DDTs, Dieldrin, Methylmercury, PCBs, Selenium, and Toxaphene. Pesticide and Environmental Toxicology Branch Office of Environmental Health Hazard Assessment, California Environmental Protection Agency.

- http://www.oehha.ca.gov/fish/gtlsv/pdf/draftGTLSVchddt.pdf

- Central Valley Regional Water Quality Control Board (CVRWQCB).  1998.  Basin Plan.  Fourth Edition, The Sacramento River Basin and the San Joaquin River Basin.  September 1998.

- TABLES

Table AQ 11-1.  Water Quality Monitoring and Sampling Locations.

| Study Reach | Water Quality Sampling Sites | Water Quality Monitoring and Sampling Locations |
| --- | --- | --- |
| Duncan Creek |  |  |
| Duncan Creek upstream of Diversion | DC-1 | Duncan Creek above diversion |
| Duncan Creek from Diversion to confluence with Middle Fork American River | DC-2 DC-3 | Duncan Creek below diversion Duncan Creek above Middle Fork American River confluence |
| Middle Fork American River |  |  |
| Middle Fork American River upstream of French Meadows Reservoir | MFAR-1 | Middle Fork American River above French Meadows Reservoir |
| French Meadows Reservoir | FM-1, FM-2, and FM-3 | Three locations in French Meadows Reservoir |
| Middle Fork American River from French Meadows to confluence with Duncan Creek | MFAR-2 MFAR-3 | Middle Fork American River below French Meadows Dam Middle Fork American River above Duncan Creek confluence |
| Middle Fork American River from confluence with Duncan Creek to Middle Fork Interbay | MFAR-4 MFAR-5 | Middle Fork American River below Duncan Creek confluence Middle Fork American River above Interbay Reservoir |
| Middle Fork Interbay | IR-1 | In Middle Fork Interbay |
| Middle Fork American River from Middle Fork Interbay to Ralston Afterbay | MFAR-6 MFAR-7 | Middle Fork American River below Middle Fork Interbay Middle Fork American River above Ralston Afterbay |
| Ralston Afterbay Downstream |  |  |
| Ralston Afterbay | RA-1 | In Ralston Afterbay |
| Middle Fork American River from Ralston Afterbay to confluence with Canyon Creek | MFAR-81 MFAR-91 | Middle Fork American River below dam Middle Fork American River below Oxbow Powerhouse tailrace |
| Middle Fork American River from confluence of Canyon Creek to confluence with North Fork American River | MFAR-101 MFAR-111 | Middle Fork American River below the Drivers Flat Road Rafting Take-Out Middle Fork American River above North Fork American River |
| North Fork American River from confluence with Middle Fork American River to Folsom Reservoir | NFAR-11 | North Fork American River below Middle Fork American River |

Table AQ 11-1.  Water Quality Monitoring and Sampling Locations (continued).

| Study Reach | Water Quality Sampling Sites | Water Quality Monitoring and Sampling Locations |
| --- | --- | --- |
| Rubicon River |  |  |
| Rubicon River upstream of Hell Hole Reservoir | RR-1 | Rubicon River above Reservoir |
| Hell Hole Reservoir | HH-1, HH-2, and HH-3 | Three locations in Hell Hole Reservoir |
| Rubicon River from Hell Hole Reservoir to confluence with South Fork Rubicon River | RR-2 RR-3 SFRR-1 | Rubicon River below dam Rubicon River above South Fork Rubicon River confluence South Fork Rubicon River above Rubicon River confluence |
| Rubicon River from confluence with South Fork Rubicon River to Ralston Afterbay | RR-4 RR-5 RR-6 RR-7 | Rubicon River below South Fork Rubicon River confluence Rubicon River above Long Canyon Creek confluence Rubicon River below Long Canyon Creek confluence Rubicon River above Ralston Afterbay |
| Long Canyon Creek |  |  |
| North Fork Long Canyon Creek upstream of Diversion | NFLCC-1 | North Fork Long Canyon Creek above diversion |
| North Fork Long Canyon Creek from Diversion to confluence with Long Canyon Creek | NFLCC-2 NFLCC-3 | North Fork Long Canyon Creek below diversion North Fork Long Canyon Creek above Long Canyon Creek confluence |
| South Fork Long Canyon Creek upstream of Diversion | SFLCC-1 SFLCC-2 | South Fork Long Canyon Creek above diversion South Fork Long Canyon Creek below diversion |
| South Fork Long Canyon Creek from Diversion to confluence with Long Canyon Creek | SFLCC-3 | South Fork Long Canyon Creek above Long Canyon Creek confluence |
| Long Canyon Creek from North and South Fork Long Canyon creeks confluence to confluence with Rubicon River | LCC-1 LCC-2 | Long Canyon Creek below North Fork and South Fork Long Canyon creeks confluence Long Canyon Creek above Rubicon River confluence |

1If peaking operations are occurring during sampling periods, water quality samples will be collected during high and low flow events.

Table AQ 11-2.  Fecal Coliform Sampling Locations Using SWRCB Protocols.

| Sample Identification Label | Sample Location |
| --- | --- |
| FC-1 | Middle Fork American River below Ahart Campground |
| FC-2 | Middle Fork American River below Gates Group Campground |
| FC-3 | Middle Fork American River below Coyote and Lewis Campground |
| FC-4 | French Meadows Reservoir near McGuire Picnic Area |
| FC-5 | French Meadows Reservoir near McGuire Boat Ramp |
| FC-6 | French Meadows Reservoir near French Meadows Campground |
| FC-7 | French Meadows Reservoir near French Meadows Boat Ramp |
| FC-8 | French Meadows Reservoir near Poppy Campground |
| FC-9 | Hell Hole Reservoir near Upper Hell Hole Reservoir Campground |
| FC-10 | Hell Hole Reservoir near Hell Hole Boat Ramp |
| FC-11 | South Fork Long Canyon Creek above Big Meadows Campground |
| FC-12 | South Fork Long Canyon Creek below Big Meadows Campground |
| FC-13 | South Fork Long Canyon Creek above Middle Meadows Campground |
| FC-14 | South Fork Long Canyon Creek below Middle Meadows Campground |
| FC-15 | Ralston Afterbay near Ralston Picnic Area |
| FC-16 | Middle Fork American River below Oxbow Powerhouse (Horseshoe Bar Area) |
| FC-17 | Middle Fork American River below the Drivers Flat Road Camping and Rafting Take-out |

Table AQ 11-3.  Parameters for Water Quality Monitoring and Laboratory Analysis.

| Parameter | Analysis Method | Sample Holding Times |  |  | Sample Locations to be Analyzed |
| --- | --- | --- | --- | --- | --- |
| Water Quality Monitoring Parameter |  |  |  |  |  |
| In-Situ Measurements |  |  |  |  |  |
| Dissolved Oxygen (DO) | Water Quality Meter |  | Not Applicable | All |  |
| Secchi Depth | Secchi Disk |  | Not Applicable | Reservoir |  |
| PH | Water Quality Meter |  | Not Applicable | All |  |
| Water Temperature | Water Quality Meter |  | Not Applicable | All |  |
| Specific Conductance | Water Quality Meter |  | Not Applicable | All |  |
| Laboratory Analysis Parameter |  |  |  |  |  |
| General Parameters |  |  |  |  |  |
| Calcium | EPA - 200.7 |  | 180 days | All |  |
| Chloride | EPA - 300.0 |  | 28 days | All |  |
| Hardness | EPA - 130.2 |  | 180 days | All |  |
| Magnesium | EPA - 200.7 |  | 180 days | All |  |
| Nitrate/Nitrite | EPA - 353.2 |  | 48 hours | All |  |
| Ammonia as N | EPA - 350.1 |  | 28 days | All |  |
| Total Kjeldahl Nitrogen | EPA - 351.2 |  | 28 days | All |  |
| Total Phosphorus | EPA - 365.2 |  | 28 days | All |  |
| Ortho-phosphate | EPA - 365.1 |  | 48 hours | All |  |
| Potassium | EPA - 200.7 |  | 180 days | All |  |
| Sodium | EPA - 200.7 |  | 180 days | All |  |
| Sulfate | EPA - 300.0 |  | 180 days | All |  |
| Total Dissolved Solids | EPA - 160.1 |  | 7 days | All |  |
| Total Suspended Solids | EPA - 160.2 |  | 7 days | All |  |
| Turbidity | EPA - 180.1 |  |  | All |  |
| TOC | EPA - 415.1 |  | 28 days | All |  |
| Total Alkalinity | EPA - 310.1 |  | 14 days | All |  |
| Metals – Dissolved |  |  |  |  |  |
| Arsenic | EPA - 1638 |  | 48 hours | All |  |
| Cadmium | EPA - 1638 |  | 48 hours | All |  |
| Copper | EPA - 1638 |  | 48 hours | All |  |
| Iron | EPA -1638 |  | 48 hours | All |  |
| Lead | EPA - 1638 |  | 48 hours | All |  |

Table AQ 11-3.  Parameters for Water Quality Monitoring and Laboratory Analysis (continued).

| Parameter | Analysis Method | Sample Holding Times |  |  |  |  | Sample Locations to be Analyzed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Laboratory Analysis Parameter (continued) |  |  |  |  |  |  |  |
| Manganese | EPA - 1638 |  | 48 hours |  | All |  |  |
| Nickel | EPA - 1638 |  | 48 hours |  |  |  |  |
| Chromium | EPA - 1638 |  | 48 hours |  |  |  |  |
| Metals – Total |  |  |  |  |  |  |  |
| Mercury | EPA - 1631e |  | 48 hours |  | All |  |  |
| Hydrocarbons |  |  |  |  |  |  |  |
| Methyl-tertiary Butyl Ether (MtBE) | EPA - 8260 |  |  | 14 days |  | Reservoir |  |
| Total Petroleum Hydrocarbons | EPA - 8020 |  |  | 14 days |  | Reservoir |  |
| Oil and Grease | EPA - 1664 |  |  | 48 hours |  | Reservoir |  |
| Bacteria |  |  |  |  |  |  |  |
| Total Coliform | EPA - SM9222B |  |  | 24 hours |  | All |  |
| Fecal Coliform | EPA - SM922B |  |  | 24 hours |  | (1) |  |

- At fecal coliform sampling locations listed in Table AQ 11-2 and depicted in Map AQ 11-2.

MAPS